Parametric Studies on Fiber Rebound in Dry-Mix Shotcrete

Vivek Bindiganavile (Contact Author: vivek@ualberta.ca)

Department of Civil & Environmental Engineering, University of Alberta, 3-020 NREF Bldg., Edmonton, AB, T6G 2W2, Canada.

Nemkumar Banthia

Department of Civil Engineering, University of British Columbia, #2024-6250 Applied Science Lane, CEME Bldg., Vancouver, B.C., V6T 1Z4, Canada

# ABSTRACT	When concrete is sprayed during shotcreting, inevitably some material is lost due to rebound. The relative losses for coarse aggregates and fibers are higher than that for the rest and therefore, the composition of the material in-place is deficient in these phases. In order to compensate for such loss and indeed to minimize it, one must identify the parameters affecting rebound and comprehend the underlying mechanisms. Recent work has established a theory of rebound for coarse aggregates. However, fibers differ from aggregates in their material and geometric properties and the effect of these must be understood in order to extend the existing theory of rebound to fibers. This is a study on the influence of particle density upon the rebound of a projectile impinging via the dry-mix process onto fresh shotcrete. The empirical model developed in this study postulates that a lower aggregate density favours lower rebound and this prediction was verified using two kinds of lightweight aggregates in dry-mix shotcrete.